SM2260

SSD Flash Controller

SSD MP APPLICATION

Confidential

Version : 1.1

Contents

Update History	- 3 -

Introduction	- 4 -

Application	- 4 -

Platform	- 4 -

Functionality	- 5 -

	Device initial page(Fig. 1)	- 5 -

Test Flow	- 6 -

Figures

Fig.1 Main Dialog	- 6 -

Fig.2 Test Device Flow	- 7 -

Fig.3 Select Configuration File	- 8 -

Fig.4 Update LightSwitch Tool	- 8 -

Fig.5 The Result of Success Testing	- 9 -

Fig.6 The Result of Fail Testing	- 9 -

Update History

| Date | Version | Description | Editor |
| --- | --- | --- | --- |
| O0707 | 1.0.0 | First version | Ginny |
| O0717 | 1.1.0 | Add to update LS settings | Ginny |
| O0723 | 1.1.1 | 1. Fix LS CRC issue 2. Modify the flow to execute LS tool 3. Show message if LS file is updated. | Ginny |
| O0724 | 1.1.2 | Add "Create Image" button | Ginny |

Introduction

This user guide provides SMI’s recommendations for Mass Production. The application’s functionality and operation flow will be introduced by this document.

介绍

该用户指南提供了SMI的批量生产建议。本文档将介绍该应用程序的功能和操作流程。

Application

The application “TestAP_SM2260.exe” named “MP Tool” services special purpose for Mass Production. The MP Tool supports 4 devices at most on system. It allows PCIe-based host system to detect and initiate a SMI SM2260 SSD Flash device and only supports one port device initialization. Also, it provides a user-friendly interface to users.

名为“ MP Tool”的应用程序“ TestAP_SM2260.exe”为批量生产提供特殊服务。 MP工具在系统上最多支持4个设备。它允许基于PCIe的主机系统检测并启动SMI SM2260 SSD闪存设备，并且仅支持一个端口设备初始化。而且，它为用户提供了用户友好的界面。

Platform

- Windows 7

Functionality

- Device initial page(Fig. 1)

Scan Drive:  An icon as  in Fig. 1. Pushing this icon will scan all drive devices on system. The status of drive icon will be changed as  if there is a SM2260 device on system.

扫描驱动器：如图1所示的图标。按此图标将扫描系统上的所有驱动器设备。就像系统上有SM2260设备一样，驱动器图标的状态也会更改。

Drive Selector: An icon as  in Fig. 1. The drive device will be selected for operation by pushing the icon and the status of icon will become enabled.

Drive Selector（驱动器选择器）：如图1所示的图标。按下该图标将选择要运行的驱动器设备，并且该图标的状态将变为启用状态。

Config List: A “ComboBox” could select the configuration which user wants to use. A “Config Save” button operation can save current settings to selected configuration.

A “Config Save As” button operation can save current settings to another file.

配置列表：“组合框”可以选择用户要使用的配置。 “配置保存”按钮操作可以将当前设置保存为所选配置。

“配置另存为”按钮操作可以将当前设置保存到另一个文件。

Flash Select: The SM2260 controller supports a lot of flashes. User should select the flash in used. A “Auto” button can automatic detect what flash on drive device.

闪光灯选择：SM2260控制器支持很多闪光灯。用户应选择使用的闪光灯。 “自动”按钮可以自动检测驱动器上的闪光灯。

Download DRAM Setting and MPISP:  The MPISP package will be loaded to SM2260 device while start test procedure. The MPISP bin file should be selected in “firmware” folder.

下载DRAM设置和MPISP：在开始测试过程时，将MPISP程序包加载到SM2260设备。 MPISP bin文件应在“ firmware”文件夹中选择。

Do Pretest Function: The flash on SM2260 device will be initiated while start test procedure.

执行预测试功能：在开始测试过程时，SM2260设备上的闪光灯将启动。

Program ISP: The ISP package will be loaded to SM2260 device and LightSwitch settings will be replaced with selected file while start test procedure. The ISP and LightSwitch bin file should be selected in “firmware” folder.

程序ISP：在开始测试过程时，将ISP软件包加载到SM2260设备，并且将LightSwitch设置替换为所选文件。 ISP和LightSwitch bin文件应在“ firmware”文件夹中选择。

Setting Item

Serial number: Select 「Update Serial」or「No Update Serial」of serial numbers.

- 「Update Serial」 policy will write the serial number of “Begin” Text Item into the SM2260 device and automatic increase serial number after test procedure completed. 「No Update Serial」policy will not enable setting serial number.

- Begin: Set the first serial number.

- End: Set the end serial number.

- Mask: The Mask item will limit the display of 「Begin」 and  「End」 items The specific character of 『#』 will be the increasing serial number set. For example, if the 「Mask」 set as 『AA###』, then the 「Begin」 will be between 『AA000』 and 『AA999』.

Other Setting Button: Pushing button can update the setting in LightSwitch bin file.

设定项目

序列号：选择序列号的“更新序列号”或“无更新序列号”。

1.“更新序列号”策略将“ Begin”文本项的序列号写入SM2260设备，并在测试过程完成后自动增加序列号。 “无更新序列号”策略将不会启用设置序列号。

2.开始：设置第一个序列号。

3.结束：设置结束序列号。

4.遮罩：“遮罩”项目将限制显示“开始”和“结束”项目。『＃』的特定字符将是递增的序列号集。例如，如果“遮罩”设置为“ AA ###”，则“开始”将在“ AA000”和“ AA999”之间。

其他设置按钮：按此按钮可以更新LightSwitch bin文件中的设置。

Start Button: Pushing button will start the initial device process.

开始按钮：按下按钮将启动初始设备过程。

Create Image Button: Pushing button will create the new ISP image with selected LightSwitch settings.

创建图像按钮：按下按钮将使用选定的LightSwitch设置创建新的ISP图像。

Fig.1 Main Dialog

Test Flow

- Step 1: Plugging the prepared testing devices into system.

- 步骤1：将准备好的测试设备插入系统。

- Step 2: Push “Scan Drive” icon for device detection and select testing device. The device icon will light up and show the device information if there is a SM2260 device plugged within. Otherwise the device icon will be murky gray.

- 步骤2：按“扫描驱动器”图标进行设备检测，然后选择测试设备。如果插入了SM2260设备，设备图标将亮起并显示设备信息。否则，设备图标将变成暗灰色。

- Step 3: Select configuration file user used from “Config List” as Fig.3

- 步骤3：从“配置列表”中选择使用的配置文件用户，如图3

- Step 4: Push “Auto” button for flash selection.

- 步骤4：按“自动”按钮进行闪光灯选择。

- Step 5: Enable “Download DRAM Setting and MPISP” and select DRAM and MPISP package bin file.

- 步骤5：启用“下载DRAM设置和MPISP”，然后选择DRAM和MPISP软件包的bin文件。

- Step 6: Enable “Do Pretest Function”.

- 步骤6：启用“执行预测试功能”。

- Step 7: Enable “Program ISP”. Select ISP package and LightSwitch settings bin file.

- 步骤7：启用“ Program ISP”。选择ISP程序包和LightSwitch设置bin文件。

- Step 8: Select Serial Number policy and set “Begin”、”End” and ”Mask” values.

- 步骤8：选择“序列号”策略并设置“ Begin”，“ End”和“ Mask”值。

- Step 9: Push "Other Setting" button to update LightSwitch settings if needs. This step will execute LightSwitch tool as Fig.4 and update the LightSwitch file that user selected in Step7.

- 步骤9：如果需要，请按“其他设置”按钮更新LightSwitch设置。该步骤将执行如图4所示的LightSwitch工具，并更新用户在Step7中选择的LightSwitch文件。

- Step10: Push “Start” button to start testing process according to the selected test items. When test procedure is completed, the progress bar will be 100% and the test result will be presented at status bar. The test result could be “Successful” as Fig.4 or some error message as Fig.

- 步骤10：按“开始”按钮，根据所选的测试项目开始测试过程。测试过程完成后，进度条将为100％，测试结果将显示在状态栏中。测试结果可能是“成功”，如图4或某些错误消息，如图4所示。

Fig.2 Test Device Flow

图2测试设备流程

Fig.3 Select Configuration File

图3选择配置文件

Fig.4 Update LightSwitch Tool

图4更新LightSwitch工具

Fig.5 The Result of Success Testing

图5成功测试的结果

Fig.6 The Result of Fail Testing

图6失败测试结果